#include <LiquidCrystal.h>

LiquidCrystal lcd(12, 11, 5, 4, 3, 2);

const byte pulsePin = 8;

const unsigned long resistance = 10000;

volatile boolean triggered;

volatile boolean active;

volatile unsigned long startTime;

volatile unsigned long duration;

ISR (ANALOG_COMP_vect)

{

unsigned long now = micros ();

if (active)

{

duration = now - startTime;

triggered = true;

digitalWrite (pulsePin, LOW);

}

}

void setup ()

{

pinMode(pulsePin, OUTPUT);

digitalWrite(pulsePin, LOW);

lcd.begin(16, 2);

lcd.print("Starting");

delay(1000);

lcd.clear();

ADCSRB = 0;

ACSR =  _BV (ACI)

| _BV (ACIE)

| _BV (ACIS0) | _BV (ACIS1);

}

void loop ()

{

if (!active)

{

active = true;

triggered = false;

digitalWrite (pulsePin, HIGH);

startTime = micros ();

}

if (active && triggered)

{

active = false;

lcd.print("Capacitance = ");

lcd.setCursor(0,1);

lcd.print(duration * 1000 / resistance);

lcd.print(" nF");

triggered = false;

delay (3000);

lcd.clear();

}

}